International Optimist Class Association (UK)

2025 Optimist Nationals – Regatta Fleet Safety Checklist

| Item | Requirement | Class Rule |
| --- | --- | --- |
| Painter | Not less than 5mm diameter. Buoyant rope. At least 8 metres long with loop tied 1m from bow. securely fastened to the mast thwart or mast step | CR4.3 (b) |
| Mast Clamp | (or alternative mechanism) to prevent mast coming out of mast step if capsized | CR3.5.2.10 |
| Buoyancy Bags | 3 x 45 litre (+/- 5 litre) bags serviceable and fixed properly as per Annual Buoyancy Test | CR3.2.7.1 |
| Bailers | 2 large bailers, each minimum 1 litre capacity. Secured to boat | CR4.3 (a) |
| Paddle | “Praddle” type is recommended. Paddle must be secured to hull with bungee or lanyard | CR4.3 (c) |
| Mainsheet | Detachable clip of recommended type which has a positive means of locking. Recommended type | IODA Safety warning |
|  | Not permitted include: |  |
| Boom span for mainsheet | Maximum gap of 100mm between boom and span rope Max 100mm | CR3.5.3.8 |
| Daggerboard | Secured to boat by bungee or lanyard | CR3.3.4 |
| Rudder retaining clip | Retaining clip must be fitted and serviceable to prevent rudder becoming detached when capsized | CR3.4.5.2 |
| Personal Flotation Device | Buoyancy Aid ISO 12402-5 (Level 50) or equivalent | CR4.2 |
| Whistle | Securely attached to buoyancy aid | CR4.2 |